Toronto Metropolitan University

# Lab Report 5:

# The Metal-Oxide-Semiconductor Field-Effect Transistor (MOSFET)

Cheng, Vincent 501173673

Mourad, Shai 501170415

PCS 224 Section 09/14

Professor X. Yuan

T.A Revin Toquero

28 November, 2023

### Introduction

The lab aims to explore MOSFET behavior using an Arduino-powered circuit by measuring drain voltages and drain current under different gate voltages. The analysis focused on understanding MOSFET responses to different voltage combinations, particularly examining drain current versus drain voltage graphs and determining threshold voltage and transconductance gain from drain current versus gate voltage data.

### Theory

A MOSFET, a type of transistor with gate, source, and drain terminals, serves as an electrically driven switch. There are two main types: depletion and enhancement, with n-channel or p-channel substrates. The lab utilizes an n-channel enhancement mode MOSFET.

Figure 1: Structure and Schematic of n-channel enhancement mode MOSFET

Figure 2: Flat band where VG = 0 volts

When VG is 0V, Fermi energy levels align, forming a "flat band." Substrate is strongly p-type, and hole concentration (p) is given by:

p=NaEF-EVKT #### Equation (1)

Figure 3: Accumulation state where VG < 0 volts

Negative charges at the gate attract holes, causing band bending. Increased charge carrier density defines the "accumulation" state. Capacitance is

Cox=ϵoxdox #### Equation (2)

Figure 4: Depletion state where VG > 0 volts

Positive charges at the gate induce depletion, reducing the p-type semiconductor. Depletion width (W) is determined by:

W=[2ϵs(ϕs)qNa]1/2 #### Equation (3)

Capacitance is:

Cd=ϵsW #### Equation (4)

Figure 5: Inversion state where VG >> 0 volts

As VG exceeds a threshold (VT), an inversion state forms an n-channel. Depletion layer depth is maximum, and total capacitance is minimum:

C=CoxCdCox+Cd #### Equation (5)

The primary objective of this lab is to observe and document the device's behavior in the inversion state as VDS undergoes variation. At small VDS, the current is directly proportional to VDS. As VDS increases, and VG surpasses VT, all charge carriers are effectively swept from drain to source. The challenge lies in measuring or calculating the current and other associated parameters.

When VGS significantly exceeds VT, the MOSFET transitions into the non-saturated region, characterized by drain current represented as:

ID=Kn[2(VGS-VTN)VDS-VDS2] #### Equation (6)

Here, the transduction parameter is:

Kn=WμnCox2L #### Equation (6)

In the linear (non-saturated) region, with very small VDS, the drain current is given by:

ID=Kn[2(VGS-VTN)VDS] #### Equation (7)

For the saturated region, the drain current is expressed as:

ID=Kn(VGS-VTN)2 #### Equation (8)

Here, the transduction parameter is:

Kn=WμnCox2L #### Equation (9)

It's important to note that this equation assumes infinite output impedance, implying that the current ID remains constant when VDS is large, making it applicable under specific conditions. The final principle of interest is the transconductance gain, given by:

gfs=∂IDS∂VGS #### Equation (10)

### Procedure

### Connect the jumper wires to the breadboard to match the I-V Curve Tracer Circuit Layout and Arduino board to the computer. Upload the Arduino code to the board and collect the ID vs VDS data using different gate voltages (VGS), beginning at 5V until 1V is reached, decreasing each trial by 1V. Plug the board into a 9V power supply when collecting data. Use the serial monitor to obtain VDS and ID values. Alter the program’s code to measure ID and VGS when  VDS is set to 5 volts. Disconnect the Arduino board and return the materials. Using the data from the serial monitor, create two graphs; one for ID vs VGS and another for ID vs VDS. The ID vs VDS graph should contain five different curves, one for each voltage of VGS. Observe the ID vs VGS graph and locate the VGS value at which the slope becomes a constant positive value. This is the VT. Calculate The Transconductance Gain (gfs). Compare and comment on the collected values to the theoretical values using a datasheet for the given transistor. Calculate a percent error to prove the accuracy of the collected data. Create a conceptual MOSFET Switch and give it a VDD and P based on one’s student ID. Find the current of the switch and then calculate the dissipated power. Find the VDS based on the previously created graphs and the found VT value. Use the transistor datasheet to make sure the switch would realistically work, by comparing the PDiss and Pmax.

### Apparatus

- MOSFET transistor (2N7000, ZVN2106, or similar)

- Arduino ( Adafruit Metro Mini, or suitable alternative version)

- Adafruit MCP4745 12-bit 5V DAC breakout board (2 required)

- Adafruit INA219 DC High-Side Current Sensor breakout board

- MCP4002 dual op-amp (CMOS)

- TIP41C BJT transistor and external power supply

- Electrical prototyping board

- Barrel jack receiver

- 9V power supply (barrel jack)

- Jumper wires

### Results and Data

| Table 1.0: Data Trends of ID and VDS for the Each VGS value |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VGS = 5.00V |  | VGS = 4.00V |  | VGS = 3.00V |  | VGS = 2.00V |  | VGS = 1.00V |  |
| VDS (V) | ID (mA) | VDS (V) | ID (mA) | VDS (V) | ID (mA) | VDS (V) | ID (mA) | VDS (V) | ID (mA) |
| 0.00 | -0.10 | 0.00 | -0.20 | 0.00 | 0.10 | 0.00 | 0.00 | 1.04 | -0.20 |
| 1.01 | 441.50 | 1.01 | 313.40 | 1.00 | 81.60 | 1.00 | 0.10 | 2.01 | 0.10 |
| 2.00 | 694.20 | 2.01 | 396.10 | 2.00 | 84.60 | 2.02 | 0.10 | 3.02 | -0.20 |
| 3.01 | 767.20 | 3.02 | 401.50 | 3.02 | 85.50 | 3.00 | 0.00 | 4.00 | -0.10 |
| 3.11 | 756.00 | 3.67 | 399.80 | 4.07 | 86.20 | 4.32 | 0.20 | 4.36 | -0.20 |

| Table 2.0: Data Trends of ID and VGS at VDS = 5.00V |  |
| --- | --- |
| VDS (V) | ID (mA) |
| 0.00 | 0.00 |
| 1.03 | 0.00 |
| 2.00 | 0.20 |
| 3.00 | 86.20 |
| 4.00 | 407.40 |
| 4.98 | 766.10 |

Figure 4.0: Graph of ID Versus VDS  for Differing Values of VGS

Figure 5.0: Graph of ID versus VGS for VDS = 5.00V

### Analysis and Calculation

By observing figure 4.0, it can be observed that an increase in VGS results in a higher slope and maximum ID. From figure 5.0, it can be seen that the threshold or turn-on voltage (VT) is where the slope becomes constant. This is around 3.00V, which means that VT = 3.00V.

| Calculating The Transconductance Gain (gfs): |  |
| --- | --- |
| gfs = ∆ID∆VGS = IDmax - IDminVGSmax - VGSmin = 766.10mA - 0.00mA4.98V - 0.00V gfs= 153.84mAV |  |

By observing figure 4.0, the resulting graph matches the theoretical outcome of the experiment. This can be true since both the slope and ID increase as VGS increases. Additionally, VGS has a lesser effect on ID as VDS increases.

By comparing the gfs on the ZVN2106 datasheet, who’s value is 300mA/V, the obtained value is relatively close to the given value. Additionally, their VT is approximately 3.5V, which is also quite close to the experimental value.

| Percent Error for The Transconductance Gain (gfs), and Threshold Voltage (VT): |  |
| --- | --- |
| Percent Error=¦c𝑎𝑙𝑐𝑢𝑙𝑎𝑡𝑒𝑑 −𝑎𝑐𝑐𝑒𝑝𝑡𝑒𝑑¦𝑎𝑐𝑐𝑒𝑝𝑡𝑒𝑑 ×100% Percent Error gfs=¦153.84mAV - 300mAV¦300mAV ×100% = 48.72% Percent Error VT=¦3.00V − 3.5V¦3.5 ×100% = 14.29% |  |

The proximity of the values obtained within the experiment is proven by the calculation of the percent error. These percent values are acceptable since the experiment handles very small values.

### MOSFET Switch Concept

Consider the circuit in figure 6.0 and assume figures 4.0 and 5.0 apply to this circuit. When the VDD = 76V and P = 3W, the resulting current will be 3W/76V = 0.0395A = 39.5mA. Based on figure 5.0, the VT = 3.00V, and on the table data for figure 4.0, the VGS = 5.00V and VDS = 0.06V. These values were chosen as they have the closest characteristics to the desired ID.

| Calculating The Power Dissipation (PDiss) |  |
| --- | --- |
| PDiss = VDSID = 0.0395A*0.06V = 2.37 x 10-3W |  |

Based on the datasheet for the ZVN2106 MOSFET transistor, the Pmax = 700mW = 0.7W. Since PDiss< Pmax, the MOSFET will survive as P will not be overloaded or destroy the circuit.

Figure 6.0: Conceptual MOSFET Switch (Dr. Xu, Toronto Metropolitan University,  MOSFET PCS224 [lab manual] - page 9 - Fall 2023)

### Discussion/Conclusion

### From the data collected within this experiment, it can be understood that VDS and VGS both have a significant effect on the current ID. They change the resulting slope, and maximum value proportionally. By conducting this experiment the theorems and key concepts of MOSFETs hold true, allowing them to be correctly used within real world technologies like vehicles. To create a smaller percent error between the calculated and actual values, it may be necessary to collect multiple sets of data for each various VGS values so that the data can be more accurate. From the experiment, the validity for the use of a MOSFET is legitimate. The MOSFET can be used as various components based on how it is set up. This allows for a single item to be used instead of multiple, lowering the complexity of a circuit.

### Appendix

| Link to Arduino Codes |  |
| --- | --- |
| https://drive.google.com/file/d/1PhrVtKkXf_bsgUI4RNFZpB-hpLaV2I_T/view?usp=drive_link https://docs.google.com/document/d/1Di7s1iSaPiQIIf9Id1S8hmPqF89lVY5hrNQHrZoirOA/edit?usp=sharing |  |

References

Dr. Yuan Xu. 2023 Fall. MOSFET PCS224 [lab manual]. Toronto (ON): Toronto Metropolitan University

Serway, R. A., & Jewett, J. W. (2018). Physics for scientists and engineers (10th ed.). Cengage Learning.

Toronto Metropolitan University. (n.d.). Physics Lab Guide [PDF]. D2L. https://courses.torontomu.ca/d2l/le/content/795518/viewContent/5278918/View

Unknown. (1994, March). N-CHANNEL ENHANCEMENT MODE VERTICAL DMOS FET. Diodes Incorporated - Analog, Discrete, Logic, Mixed-Signal. https://www.diodes.com/assets/Datasheets/ZVN2106A.pdf